Annual emissions reporting

# INTRODUCTION

Reporting of greenhouse gas (GHG) emissions from installation which falls under the scope of European Union Emissions Trading System (EU ETS) requirement is laid down in Commission Regulation No. 601/2012. According to article 67 of this monitoring and reporting regulation (MRR) each operator has to submit an annual emissions report (AER) by 31 of March each year for the reporting period (for example in 2015 operators will have to submit an annual emissions report for 2014).

Annual emissions report contains relevant information about the operator and the installation that is managed by the operator, information on activities carried out in the installation that fall under the scope of EU ETS, information on emissions from source streams and emission sources and other relevant information.

Let us overview reporting requirements on emission sources and source streams.

# Reporting requirements on emission sources and source streams

Regulation 601/2012 defined the minimum reporting requirements in the annex X. These requirements represented the minimum mandatory information that needs to be submitted by each operator. This led to greater harmonization and consistency of reported information. These requirements can be a bit different according to whether the operator is reporting emissions from emission sources or emissions from source streams and methodologies used for reporting but the main principle stays the same: minimum information which enables consistent, accurate and transparent submission of information.

So Annex X lays down that information, reported in the AER for all emissions sources and source streams, shall at least include:

- whether the measurement or the calculation methodology is applied;

- Activity data (based on fuel supplier documentation or determined by operators measurement instruments):

- in the case of fuels the amount of fuel (expressed as tonnes or Nm3) and the net calorific value (GJ/t or GJ/Nm3) reported separately;

- for all other source streams the amount expressed as tonnes or Nm3;

- emission factors (t CO2/TJ, t CO2/t, t CO2/Nm3) and biomass fraction, oxidation and conversion factors (expressed as dimensionless fractions);

- Emissions expressed as tonnes of CO2 equivalent (t CO2(e)). This means that if not only CO2 is emitted from the installation, the sum of all GHG emitted should be expressed as t CO2(e);

- where greenhouse gases other than CO2 are emitted, the total emissions expressed as tonnes;

- Tiers applied. A “tier” means a set requirement used for determining activity data, calculation factors, annual emission and annual average hourly emission. Tiers for activity data are ranging from 1 to 4 and their uncertainty level decreases accordingly from 7,5% to 1,5%. For emission factors, net calorific values and carbon content tiers range from 1 to 3 and are defined as follows:

- Tier 1: Standard factors based on IPCC 2006 (values given in Annex VI of MRR),

- Tier 2(a/b): Country specific standard factors from national GHG inventories,

- Tier 3: Values determined by chemical analysis.

# Annex X of reporting requirements:

## data gaps

There are times when some data relevant to determination of GHG emissions might not be available or missing. According to MRR in this situation the operator shall use an appropriate estimation method for determining conservative surrogate data for the respective time period and missing parameter. These estimation methods may be based according to historical data, data trends, data parameters correlation etc.

After data gaps have been closed by surrogate data in accordance to article 65 of MRR the following information must be submitted in annual emissions report:

- Source stream or emission source to which each data gap applies;

- Reasons for each data gap. Why or how they occurred (e.g. failure of measurement instrument);

- the starting and ending date and time of each data gap;

- the emissions calculated based on surrogate data;

- where the estimation method for surrogate data has not yet been included in the monitoring plan, a detailed description of the estimation method including evidence that the methodology used does not lead to an underestimation of emissions for the respective time period;

## Memo Items

There are situations when using biomass is more economical for the operator and may result in biomass use rather than just natural gas or coal etc. For example in Lithuania biomass use for centralized heating was promoted and subsidize which resulted in strong GHG emission decrease in 2013. Although emission factor for combusting biomass is 0 t CO2/TJ, relevant information on biomass use must be submitted in AER as memo items. This is just one example of memo item. Other memo items are related to CO2 transfer etc.

So the information to be reported as memo items under the MRR requirements must at least contain the following information:

- amounts of biomass combusted (TJ), or employed in processes (t or Nm3);

- CO2 emissions from biomass (t CO2), where measurement-based methodology is used to determine emissions;

- a proxy for the net calorific value of the biomass source streams used as fuel, where relevant;

- amounts and energy content of bioliquids (TJ) and biofuels combusted (t);

- CO2 transferred to an installation or received from an installation (t CO2);

- inherent CO2 transferred to an installation or received from an installation (t CO2);

- where applicable, the name of the installation and its identification code as recognized in accordance with Regulation (EU) No 389/2013:

- of the installation(s) to which CO2 is transferred;

- of the installation(s) from which CO2 is received;

- transferred CO2 from biomass (t CO2).

To sum up a point on CO2 transfer it should be noted that reporting requirements apply only if EU ETS installation is:

- receiving inherent CO2,

- exporting inherent CO2 to another EU ETS installation,

- exporting inherent CO2 to non EU ETS installation,

- receiving transferred CO2,

- exporting transferred CO2.

# Demonstration of the Annual emissions report template

Annual emissions report template (AER) was originally designed and released by European Commission to help operators from different countries to have the same GHG reporting forms which would ensure a harmonized data submission to local competent authority (CA). This enables to have a standard form for each operator to submit required information under the MRR requirements no matter in which country the activity is carried out.

The start of demonstration of the AER will include a simple example of heat power plant which produces heat for local households. Let’s say its installed capacity is 35 MW and during the previous year it combusted 9000 tonnes of peat briquettes to produce heat. This installation will be a small category (A) installation which is considered a small emitter.

When you first open up a excel template you see the front page in which table of content, basic information (reporting year, installation name and unique identifier) is presented. These empty information fields are filled automatically as you enter data in next sheets of the template. The only thing you need to do here is to enter the date on which the AER is submitted and the name and signature of responsible person. This is done when excel sheets are printed and submitted by hard copy to the competent authority. Of course other reporting tools may be used for submitting AER (e.g via e-mail or by using IT systems).

Next sheet is called “Guidance and conditions”. This sheet gives the user relevant information on the legal basis of reporting requirements, general guidance on how to use this AER template and what are the next steps after finishing filling in the required data.

Next sheet is called “Identification of the operator, installation and verifier”. This sheet is basically needed to submit contact information of the operator to help understand where the operator and the installation are located, who is the competent authority to which this AER must be submitted etc. There is also a field where the verifiers contact information must be submitted. So I’ve entered here the required information about the operator, the installation and the verifier. One specific moment that I would like to mention is that section 3 (c) required to state if the installation has to report to European Pollutant Release and Transfer Register (EPRTR) under the regulation 166/2006. This register provides information on the amounts of pollutant released to air, water and land as well as off-site transfers of waste and of pollutants in waste water from a list of 91 key pollutants including heavy metals, pesticides, greenhouse gases and dioxins for years 2007 onwards.

Our power plant has an installed capacity of 35 MW, therefore it does not fall under EPRTR activity requirements, and so we will leave these fields empty.

Next we move on to installation description sheet. First thing we need to do here is to identify under which Annex I activity our installation falls under. As this is a heat power plant which combust fuel to generate heat, so it falls under “Combustion of fuels”.

Next we need to enter information on monitoring approach. This time we will be using calculation based approach for CO2 emissions. We can see that the template automatically shows us the relevant sections of this template which we will need to further fill in. Next we must identify our source streams in section 7 (b). Here we input information on our heat power plant:

- Source stream type: Combustion: solid fuels (peat is not considered as commercial standard fuel and therefore we identify it as solid fuel,

- Source stream category: Solid - peat.

- Source stream name: Peat briquettes.

Next we move on to “Emissions from source streams” sheet. This sheet will be the main data entry sheet for calculating CO2 emissions from natural gas combustion. Here we will enter data on amount of fuel combusted, its net calorific value (NCV), oxidation factor, emission factor (EF) etc. Let us start by entering data in the first field.

We can see from the drop down menu that we only have on possible solution to choose from while entering data on CO2 emissions. This is because we entered data only on one source stream in the previous stream. If we entered additional source streams then we would have to choose from several sources. Next we enter activity data (AD) – 9000 tonnes of peat briquettes. From tier requirements we now that category A installation using peat as fuel needs to apply following tiers:

- Activity data – Tier 1,

- EF – Tier 2a/2b,

- NCV – Tier 2a/2b,

- Oxidation factor – Tier 1.

After entering the data the template automatically calculates CO2 emissions from peat combustion. As we can see installation is indeed category A low emitter as its emissions are lower than 25 000 t CO2.

Next sheet is needed to report emission from emissions sources by using measurement based approach. Usually N2O emissions have to be determined using a measurement based approach (for example in nitric acid production in chemical industry). This sheet may also be used for reporting Transferred/inherent CO2 emissions as these are also usually determined by using measuring equipment.

(Note: Include demonstration of N2O reporting example from local nitric acid producer if enough time remaining)

Next sheet is required only if fall-back approach is used in the installation. Here an operator has to submit a description of fall-back approach, name relevant source streams to which this fall-back approach applies and also total determined GHG emissions.

Next sheet is required to report on perfluorocarbon (PFC) emissions from primary aluminum production.

Next sheet is for data gaps reporting. If an operator identified any data gaps during the reporting year he must report these in this section of the AER. As mentioned before an operator has to submit information on source streams to which data gaps were identified, time period when data gaps occurred, provide description and reasons for data gaps and what methods were used to close these data gaps. In the end an operator has to report on estimated emissions using previously described method for data gaps closing.

Finally, filling in the report is coming to an end and we find a sheet for “Further information”. This sheet is not mandatory and an operator may chose if he finds a need to provide information on its product details, used abbreviations in previous sheets and additional information if it is relevant to the reporting requirements.

After that comes “Summary” sheet were a summary of entered data in AER is presented. In this sheet you can see total emissions from the installation,  energy content of fossil fuel used, emissions from biomass (if any), reported memo items etc. A more detail information on reported GHG emissions you can see in the final AER template sheet.

# The improvement report and follow-up actions

According to MRR article 69 each operator shall regularly check whether the monitoring methodology applied can be improved and submit an improvement report to competent authority (CA). Improvement reports have to be submitted in the following order:

- for a category A installation, by 30 June every four years;

- for a category B installation, by 30 June every two years;

- for a category C installation, by 30 June every year;

- or a later date if it is set by Competent Authority, but no later than 30 September.

Also an improvement report must be submitted as a response to findings in the Verification report (VR).

A couple of reasons why an IR must be submitted:

- Verification report submitted alongside with annual emission report to CA states outstanding non-conformities or recommendations for improvement;

- When an operator applies fall-back approach. The report must contain a justification as to why it is technically not feasible or would incur unreasonable costs to apply at least tier 1 for one or more major or minor source streams. If this justification is not applicable any more, the operator has to report how at least tier 1 for those source streams is to be applied.

- When an operator does not apply minimum required tiers laid down in MRR (improvement related to GHG measurements or specific source stream).

So when a VR identifies possible improvements an operators has to:

- Send a report on the proposed improvements to the competent authority for approval,

- Update the monitoring plan as appropriate if it is required,

- Implement the improvements according to the time table proposed in the approved improvement report.

(Short presentation of improvement report template on remaining time)