|  | Avoid | Reduce | Remedy/ Compensate |
| --- | --- | --- | --- |
| Air and climate |  | - Park and ride scheme to get workers to the site. - Materials delivered by sealed/sheeted carriers. - Bunded and/or sealed storage of materials. - Minimise unnecessary handling of materials. - Use of sprinklers to reduce dust. - Wheel wash for vehicles leaving site. - Minimise HGV and plant movements on unimproved surfaces. - Speed limit of 10mph on work site. - Unloading of vehicles from lowest practical height (to minimise drop heights) and in areas protected from wind. - Direct exhaust systems for all plant away from ground. - Regular sweeping of access roads. - Vehicle engines should not be left running unnecessarily. - Locate dust generating activities away from and/or downwind of sensitive receptors. - Make use of accommodation in Combe Down. - Selection of in-fill material that minimises no. of vehicle movements. |  |
| Flora and Fauna | - Fencing-off tree-protection zones to prevent soil compaction. - Use of bat exclusion techniques to keep them out of areas to be infilled. | - Limit disturbance of bats by phasing work, restricting work at crucial times, creating sound and light barriers around key sites, and providing unworked zones. - Restrict passage of work traffic past key (above-ground) sites. - Use of absorbent flooring around key sites to reduce vibration from construction traffic. - Use of dust barriers around construction activities. - Minimise overall landtake, habitat disturbance, energy use, emission of air pollutants, water use and site run-off. - Use of inert stabilisation material to minimise long-term effect on ecology. | - Re-creation and enhancement of bat breeding and roosting habitat (e.g. re-excavation of passages and chambers, improve thermal regime by opening new ventilation shafts, provision of incubator chambers, etc). - Enhancement of bat feeding habitat surrounding the mines. - Management of retained open space around worksite to enhance biodiversity. - Restoration of biodiversity on the work site post-stabilisation. - Replace felled trees |

| Noise and vibration | - Use of alternatives to audible vehicle reversing warnings. - Avoid working at night under properties where level of overburden is minimal. | - Park and ride scheme for workers. - Phased delivery system for construction traffic and establishment of core delivery times. - Delivery vehicle routing to increase distance between sensitive receptors. - Development of an underground worksite, fitted with heavy doors. - Limiting nature of activities undertaken at night time. - Site design to reduce noise and vibration. - Production of a works method statement by appointed contractor to show how they will minimise noise. - Communication with community about work timings. - Disciplinary action for staff who breach ‘good neighbour’ policies. - Selection of infill material that considerably reduces no. of vehicle movements. |  |
| --- | --- | --- | --- |
| Cultural heritage | - ‘Preservation’ of critical resources in sand instead of concrete. In theory, the relics could be retrieved at a later date. - Fencing-off war memorial. | - Cover significant resources with geotextile membrane prior to infilling. | - Preserve resources by written, drawn and photographic record. - Creation of a GIS-based record of the system, including a ‘fly through’ video. - To prepare an archive report and make the results publicly available. - Further dissemination of findings through a web-site. - Opening of an interpretation centre containing saved relics. |
| Landscape | - Development of an underground worksite. | - Erection of 2.4m high, dark green fence around construction compound. - Restrict height of temporary buildings and stored materials to height of perimeter fence. - Use of horizontal (rather than vertical) cement silos. - Restrict lighting height and use of cut-off lighting. - 24 hour working to reduce project timescale. | - Build stone wall to replace an existing wire fence. - Replace felled trees. - Reinstate area occupied by work site post-stabilisation. |

| Socio-economic | - Workers to sign a form stating they won’t park around the site as part of their conditions of engagement. | - Provision of accurate information and timely consultation. - Maximum recruitment of non-specialist labour from within daily commuting distance of the project. - Make use of accommodation in Combe Down. - Use of local suppliers to reduce transport needs and to provide benefits to local economy. - Implementation of health and safety measures for workers. - Materials delivered and staff changeover times outside peak hours to reduce congestion. - Visit schools to warn them about activities on the site. - Prior identification of ‘pull in’ areas to which delivery vehicles can be routed if necessary. | - Relocation of children’s playground. - Upgrading play area equipment. - Provide better quality, dog-proof fencing for the play area. - Encourage use of unoccupied or under-occupied rental accommodation. - Full reinstatement of work site post-stabilisation. - Ameliorate public safety risk associated with a mine collapse. - Bring house prices and insurance premiums in line with market average. - Provision of a ‘hotline’ contact no. for public in cases where they are concerned about traffic. |
| --- | --- | --- | --- |