Impact Mitigation in Environmental Assessment

Case Study – Combe Down Stone Mines Stabilisation Programme

Task

During this session you will develop impact mitigation measures for a case study development – the Combe Down Stone Mines Stabilisation Programme.  This development involves the stabilisation of disused mines underlying the village of Combe Down, which is located on the outskirts of Bath in Somerset.  The mines present a significant health and safety risk as there is an imminent threat of collapses occurring in a number of areas.  A number of council-owned residences have had to be evacuated and weight restrictions have been imposed on traffic using the surrounding road network.   It is estimated that the mine void is 400,000m2 in size and that between 80 to 90% of this void space will need to be stabilised.  Most areas will be stabilised by filling them with concrete.  The development has a budget of more than £155 million and will take approximately four years to complete.

The stabilisation work will cause a variety of adverse impacts.  In addition to such issues as noise, vibration, dust and visual intrusion associated with the construction activities, the stabilisation scheme will impact upon internationally important ecological resources – bats and their roosting and breeding habitat.  The disused mines are also considered a nationally, and arguably internationally, significant cultural resource for these workings were the first to use a number of innovative quarrying techniques.  In the medium term, positive socio-economic impacts will accrue as the threat of collapse is attenuated and house and insurance prices come back in-line with market averages.  An overview of the principal impacts for a select range of environmental components is included in Table 1.

You should work in small groups (i.e. 3 to 4 people) when undertaking this exercise.  You will be given approximately 30 minutes to develop your proposals for impact mitigation.  Use the matrix in Appendix 1 to structure your analysis and record your suggestions for impact mitigation.  At the end of the exercise, you will be given a summary of the impact mitigation measures which were proposed in practice for this development.

Table 1 - Overview of potential impacts for selected environmental components

| Environmental component | Potential impacts |
| --- | --- |
| Air and climate | - Air and dust pollution from construction traffic (maximum of 80 HGV movements a day). - Dust from construction activities (e.g. unloading lorries, etc). - CO2 emissions associated with the use of large quantities of cement for infilling mines. |
| Flora and Fauna | - Damage (i.e. infilling) to bat breeding and roosting habitat. Bats and their habitat are protected under European legislation. - Small-scale damage to vegetation (e.g. due to tree and shrub clearance) around construction site. |
| Noise and vibration | - Noise and vibration generated by construction traffic and construction activities (e.g. operation of plant). - Stabilisation will allow heavy vehicles to use the neighbouring road system. At present, there are weight limits in place on these roads. |
| Cultural heritage | - Damage to mine relics (e.g. evidence of mining techniques employed, mine graffiti, etc.) of national, arguably international, significance. Mine works are part of a larger UNESCO World Heritage Site. - Damage to above-ground war memorial and its setting. |
| Landscape and visual | - Impacts of construction traffic and above-ground activities on an Area of Outstanding Natural Beauty (i.e. regionally significant). - Visual impacts and light pollution for residences neighbouring the construction site. |
| Socio-economic | - Loss of recreational land (i.e. children’s playground and village green) for development of construction site. - Small influx of construction workers. - Community disruption from traffic, noise and dust. - Temporary relocation of a footpath. - Bring house and insurance prices in-line with average values once stabilisation work is completed. - Alleviation of risks to health, safety and property associated with a mine collapse. Possible impacts on well-being. |

Appendix 1

Matrix for recording impact mitigation proposals

|  | Avoid | Reduce | Remedy/ Compensate |
| --- | --- | --- | --- |
| Air and climate |  |  |  |
| Flora and Fauna |  |  |  |
| Noise and vibration |  |  |  |
| Cultural heritage |  |  |  |
| Landscape |  |  |  |
| Socio-economic |  |  |  |

Due to time constraints, we will not consider the full-spectrum of environmental components during this session.